ENV.MONITOR

Real-Time IoT Environmental Monitoring System

Aim:

To develop a real-time IoT system for monitoring temperature, humidity, and air quality with secure, low-latency data visualization.

Introduction:

Indoor environmental conditions significantly affect health and comfort. ENV.MONITOR uses IoT sensors and MQTT protocol to provide real-time monitoring and visualization through a web dashboard.

Parameters Monitored:

- Temperature (°C)

- Humidity (% RH)

- Air Quality (ppm)

- AQI (via API)

System Architecture:

- ESP8266 → Collects sensor data

- MQTT (TLS) → Sends data to cloud

- HiveMQ Cloud → Broker

- Next.js Dashboard → Displays live data

- APIs → Outdoor comparison

Working Flow:

Sensor → ESP8266 → MQTT → HiveMQ → Dashboard

Tech Stack:

ESP8266, DHT11, MQ-135, OLED
 Next.js, React, mqtt.js
 MQTT, TLS, WebSockets
 HiveMQ Cloud, Vercel
 Open-Meteo API, BigDataCloud API

Key Features:

✔ Real-time monitoring
 ✔ Secure communication
 ✔ Indoor vs outdoor comparison
 ✔ Low-cost & scalable

Future Scop:

AI prediction, mobile app, smart automation, advanced sensors

Conclusion:

The system successfully enables real-time environmental monitoring using MQTT, improving awareness and supporting smart living.

Sensors:

DHT11

- Temperature & humidity

- Digital output
 👉 Low-cost, moderate accuracy

MQ-135

- Detects gases (CO₂, smoke, NH₃)

- Analog output
 👉 Air quality monitoring

OLED (SSD1306)

- 128×64 display (I2C)
 👉 Shows live data

Communication

- MQTT → Lightweight, real-time

- TLS → Secure transmission

- WebSockets → Live dashboard updates

Importance

- Temperature → Comfort

- Humidity → Respiratory health

- Air Quality → Pollution impact

👉 Indoor air can be more polluted than outdoor

References

Environmental:

- WHO (2018, 2021) – Health & Air Quality

- EPA – Indoor Air

Sensors:

- Feresu (2022) – DHT11

- Doyan et al. (2023) – Comparison

- Chenchireddy (2022), Wibowo (2024) – MQ Sensors

IoT:

- Atmoko (2017), Kashyap (2018), Gavrilov (2022) – MQTT